仿真报告

基于 Cox & Reudink (1973) 论文，完整呈现动态信道重分配系统的 MATLAB 仿真实现，包含仿真结果、代码逻辑解析及完整源代码，保障仿真可复现、原理可理解、结果可验证。

# 一、仿真结果展示（对应原文 Fig. 3-8 & Table I）

仿真复刻原文实验设置，运行后生成6类核心性能图表及1个对比表格，涵盖承载话务量、呼损率、动态信道调用、信道切换、占用率等关键指标，以及纯固定信道系统（FIXSYS）与混合信道系统（8固定+2动态）的性能对比，所有图表与原文指标定义、趋势一致，可直接用于结果验证。

# 二、代码逻辑分段详细说明

仿真代码采用模块化设计，按“初始化-预处理-核心仿真-结果输出”流程组织，各模块功能独立、逻辑连贯，具体说明如下：

## 1. 初始化与参数设置（main 函数开头）

定义仿真全局参数，包括网格尺寸、信道复用距离、总信道数、固定信道数及话务负载范围，设定呼叫时长、仿真预热与稳态统计时间，初始化随机数种子和性能指标存储矩阵，确保参数与原文一致、仿真结果可复现。

## 2. 干扰矩阵构建（build_interference_list 函数）

预处理覆盖区域间的干扰关系，基于网格坐标计算并生成每个区域的干扰列表，同时定义动态信道范围，为后续动态信道分配提供快速查询依据，提升仿真效率。

## 3. 主循环与策略遍历（main 函数中双重循环）

遍历所有固定信道数与话务负载组合，计算对应场景下的全局呼叫到达率，调用核心仿真函数批量获取呼损率、承载话务量等5类性能指标，并存储至矩阵便于后续绘图分析，实现多场景仿真对比。

## 4. 核心仿真引擎（run_sim 函数）

采用事件驱动模型，模拟呼叫从到达、分配、服务到终止的全流程；定义中心区域以避免边界效应，生成泊松分布的呼叫到达事件和指数分布的终止事件，通过优先队列调度事件，区分预热与稳态阶段统计性能数据，确保数据有效。

## 5. 信道分配与 RING 策略（run_sim 函数核心逻辑）

实现原文规定的信道分配优先级与动态重分配策略，呼叫优先使用固定信道，固定信道满则分配满足干扰约束的动态信道，无可用信道则呼叫阻塞；固定信道释放时，将区域内动态信道呼叫切换至固定信道，同时模拟车辆跨区域移动的信道重新分配逻辑。

## 6. 结果绘图与 Table I 生成（plot_all 函数 + main 函数结尾）

将仿真数据可视化，生成与原文一致的6幅性能曲线及Table I对比表格，通过插值法和Erlang B公式量化混合系统的信道节省优势；同时在命令行输出详细仿真数据表，便于精准查看参数组合对应的性能指标。

# 三、完整 MATLAB 仿真源代码（v.m）

matlab function main() clear; clc; close all; rng(42); total_channels=160; reuse_dist=4; grid_size=27; fixed_caps=[0,5,6,7,8,9,10]; load_points=4:2:18; call_duration_avg=98; warmup_time=120; sim_time=420; total_time=warmup_time+sim_time; R=zeros(5,length(fixed_caps),length(load_points)); fprintf('Building interference matrix for %dx%d grid...\n',grid_size,grid_size); [interf_cells, ring_channels] = build_interference_list(grid_size, reuse_dist); for f_idx=1:length(fixed_caps) Nf=fixed_caps(f_idx); Nd=total_channels-Nf*(reuse_dist^2); fprintf('\n=== Nf=%d (Nd=%d) ===\n',Nf,Nd); for L_idx=1:length(load_points) traffic_load=load_points(L_idx); lambda_global=(traffic_load/call_duration_avg)*(grid_size^2); [R(1,f_idx,L_idx),R(2,f_idx,L_idx),R(3,f_idx,L_idx),R(4,f_idx,L_idx),R(5,f_idx,L_idx)]=... run_sim(lambda_global,grid_size,interf_cells,ring_channels,Nf,Nd,call_duration_avg,total_time,warmup_time); fprintf(' Load %.0f -> Block:%.1f%%, Carried:%.2f, DynAcc:%.1f%%, Switch:%.1f%%, Occ:%.1f%%\n',... traffic_load,R(1,f_idx,L_idx),R(2,f_idx,L_idx),R(3,f_idx,L_idx),R(4,f_idx,L_idx),R(5,f_idx,L_idx)); end end % Compute Table I data for plotting tableI_data = []; idx_fixed=find(fixed_caps==10,1); idx_hybrid=find(fixed_caps==8,1); if ~isempty(idx_fixed) && ~isempty(idx_hybrid) target_blocking=[1,2,5]; fprintf('\n===== TABLE I =====\nBlock%% ¦ Channels Required (FIXSYS) ¦ (Hybrid: F=8+2 dynamic)\n'); for tb=1:3 blk_hybrid = squeeze(R(1,idx_hybrid,:)); carried_hybrid = squeeze(R(2,idx_hybrid,:)); [blk_unique,ia_unique]=unique(blk_hybrid,'stable'); carried_unique=carried_hybrid(ia_unique); carried_at_target=interp1(blk_unique,carried_unique,target_blocking(tb),'linear','extrap'); A=carried_at_target; Bt=target_blocking(tb)/100; Cr=1; while true num=A^Cr/factorial(Cr); den=0; for i=0:Cr, den=den+A^i/factorial(i); end if num/den<=Bt ¦¦ Cr>30, break; end Cr=Cr+1; end tableI_data = [tableI_data; target_blocking(tb), Cr]; fprintf(' %2d%% -> %2d\n',target_blocking(tb),Cr); end fprintf('注: 混合系统8固定+2动态, 纯固定系统需更多信道达相同承载\n'); end plot_all(load_points,fixed_caps,R,tableI_data); fprintf('\n\n===== 详细仿真数据表 =====\nNf\tLoad\tBlock%%\tCarried\tDynAcc%%\tSwitch%%\tOcc%%\n'); for f_idx=1:length(fixed_caps) for L_idx=1:length(load_points) fprintf('%d\t%.0f\t%.1f\t%.2f\t%.1f\t\t%.1f\t\t%.1f\n',... fixed_caps(f_idx),load_points(L_idx),R(1,f_idx,L_idx),R(2,f_idx,L_idx),R(3,f_idx,L_idx),R(4,f_idx,L_idx),R(5,f_idx,L_idx)); end end end function [blocking_pct,carried_per_chan,dyn_attempts_pct,switched_pct,occupancy_pct]=... run_sim(lambda_global,grid_size,interf_cells,ring_channels,Nf,Nd,call_duration_avg,total_time,warmup_time) num_cells=grid_size^2; central_start = floor((grid_size-15)/2) + 1; central_end = central_start + 14; central_cells = false(num_cells, 1); for i = 1:grid_size for j = 1:grid_size if i >= central_start && i <= central_end && j >= central_start && j <= central_end idx = (i-1)*grid_size + j; central_cells(idx) = true; end end end num_central = 225; fixed_in_use=zeros(num_cells,1); cell_dyn=cell(num_cells,1); for i=1:num_cells, cell_dyn{i}=false(Nd,1); end stat_arr=0; stat_blk=0; stat_dyn=0; stat_sw=0; stat_car=0; ga_central=0; max_ev=1000000; eq=zeros(max_ev,5); eqc=0; n_arr=min(ceil(total_time*lambda_global*1.5),500000); arr_t=cumsum(exprnd(1/lambda_global,n_arr,1)); arr_c=randi(num_cells,n_arr,1); ai=1; eqc=1; eq(1,:)=[arr_t(1),1,arr_c(1),0,0]; ai=2; ct=0; ga=0; while eqc>0 && ct<total_time evt=eq(1,:); eq(1,:)=eq(eqc,:); eqc=eqc-1; idx=1; while true l=2*idx; r=2*idx+1; s=idx; if l<=eqc && eq(l,1)<eq(s,1), s=l; end if r<=eqc && eq(r,1)<eq(s,1), s=r; end if s==idx, break; end tmp=eq(idx,:); eq(idx,:)=eq(s,:); eq(s,:)=tmp; idx=s; end t=evt(1); if t>total_time, break; end dt=t-ct; ct=t; is_stat=(t>=warmup_time); if is_stat, stat_car=stat_car+ga_central*dt; end if evt(2)==1 cid=evt(3); if central_cells(cid), stat_arr=stat_arr+1; end served=false; if fixed_in_use(cid)<Nf fixed_in_use(cid)=fixed_in_use(cid)+1; served=true; ch_t=1; ch_i=0; elseif Nd>0 ch_assigned = -1; for ch = 1:Nd available = true; if cell_dyn{cid}(ch), available = false; end if available for k=1:length(interf_cells{cid}) if cell_dyn{interf_cells{cid}(k)}(ch), available = false; break; end end end if available, ch_assigned = ch; break; end end if ch_assigned >= 0 cell_dyn{cid}(ch_assigned)=true; served=true; ch_t=2; ch_i=ch_assigned; if is_stat && central_cells(cid), stat_dyn=stat_dyn+1; end end end if served ga=ga+1; if central_cells(cid), ga_central=ga_central+1; end dur=max(1,round(exprnd(call_duration_avg))); eqc=eqc+1; eq(eqc,:)=[t+dur,2,cid,ch_t,ch_i]; idx=eqc; while idx>1 p=floor(idx/2); if eq(p,1)>eq(idx,1), tmp=eq(p,:); eq(p,:)=eq(idx,:); eq(idx,:)=tmp; idx=p; else break; end end elseif is_stat && central_cells(cid), stat_blk=stat_blk+1; end if ai<=n_arr && arr_t(ai)<=total_time eqc=eqc+1; eq(eqc,:)=[arr_t(ai),1,arr_c(ai),0,0]; idx=eqc; while idx>1 p=floor(idx/2); if eq(p,1)>eq(idx,1), tmp=eq(p,:); eq(p,:)=eq(idx,:); eq(idx,:)=tmp; idx=p; else break; end end ai=ai+1; end else cid=evt(3); ch_t=evt(4); ch_i=evt(5); ga=ga-1; if central_cells(cid), ga_central=ga_central-1; end if ch_t==1 fixed_in_use(cid)=fixed_in_use(cid)-1; ad=find(cell_dyn{cid},1); if ~isempty(ad) cell_dyn{cid}(ad)=false; fixed_in_use(cid)=fixed_in_use(cid)+1; for q=1:eqc if eq(q,2)==2 && eq(q,3)==cid && eq(q,4)==2 && eq(q,5)==ad eq(q,4)=1; eq(q,5)=0; break; end end if is_stat && central_cells(cid), stat_sw=stat_sw+1; end end elseif ch_i>0, cell_dyn{cid}(ch_i)=false; end end end sd=max(ct-warmup_time,1); blocking_pct=(stat_blk/max(stat_arr,1))*100; carried_per_chan=(stat_car/sd)/num_central; dyn_attempts_pct=(stat_dyn/max(stat_arr,1))*100; switched_pct=(stat_sw/max(stat_arr,1))*100; occupancy_pct=(carried_per_chan/10)*100; end function [interf_cells, ring_channels] = build_interference_list(grid_size, reuse_dist) num_cells=grid_size^2; interf_cells=cell(num_cells,1); [X,Y]=meshgrid(1:grid_size,1:grid_size); coords=[X(:),Y(:)]; min_d2=reuse_dist^2; for i=1:num_cells nb=[]; for j=1:num_cells if i==j, continue; end dx=coords(i,1)-coords(j,1); dy=coords(i,2)-coords(j,2); if dx^2+dy^2<min_d2, nb(end+1)=j; end end interf_cells{i}=nb; end Nd_max = 160; ring_channels = cell(1, 1); ring_channels{1} = 1:Nd_max; end function plot_all(load_points,fixed_caps,R,tableI_data) set(0,'DefaultAxesFontSize',10,'DefaultTextFontSize',10,'DefaultLineLineWidth',1.5); n_plots=length(fixed_caps); co=lines(n_plots); mk={'o','s','d','^','v','p','h'}; pl_str=arrayfun(@(x) num2str(x), fixed_caps, 'UniformOutput', false); figure('Name','Paper Results','Position',[50,50,1400,900]); subplot(3,3,1); hold on; grid on; for k=1:n_plots plot(load_points,squeeze(R(2,k,:)),['-' mk{mod(k-1,length(mk))+1}],'Color',co(k,:),'MarkerSize',6,'MarkerFaceColor',co(k,:),'DisplayName',sprintf('Nf=%s',pl_str{k})); end xlabel('Offered (Erl/cell)'); ylabel('Carried (Erlangs/cell)'); title('Fig3a: Carried vs Offered'); legend('Location','northwest'); box on; subplot(3,3,2); hold on; grid on; for k=1:n_plots semilogy(load_points,squeeze(R(1,k,:)),['--' mk{mod(k-1,length(mk))+1}],'Color',co(k,:),'MarkerSize',6,'DisplayName',sprintf('Nf=%s',pl_str{k})); end xlabel('Offered (Erl/cell)'); ylabel('Blocking (%)'); title('Fig3b: Blocking vs Offered'); set(gca,'YScale','log'); ylim([0.1,100]); legend('Location','northwest'); box on; subplot(3,3,3); hold on; grid on; for k=1:n_plots plot(squeeze(R(1,k,:)),squeeze(R(2,k,:)),['-' mk{mod(k-1,length(mk))+1}],'Color',co(k,:),'MarkerSize',6,'MarkerFaceColor',co(k,:),'DisplayName',sprintf('Nf=%s',pl_str{k})); end xlabel('Blocking (%)'); ylabel('Carried (Erlangs/cell)'); title('Fig4: Carried vs Blocking'); legend('Location','southeast'); box on; subplot(3,3,4); hold on; grid on; for k=1:n_plots if fixed_caps(k)>0 fc=squeeze(R(2,k,:)).*(1-squeeze(R(3,k,:))/100); plot(squeeze(R(1,k,:)),fc,['-' mk{mod(k-1,length(mk))+1}],'Color',co(k,:),'MarkerSize',6,'MarkerFaceColor',co(k,:),'DisplayName',sprintf('Nf=%s',pl_str{k})); end end xlabel('Blocking (%)'); ylabel('Fixed Carried (Erlangs/cell)'); title('Fig5: Fixed Ch Carried'); legend('Location','best'); box on; subplot(3,3,5); hold on; grid on; for k=1:n_plots if fixed_caps(k)>0 && fixed_caps(k)<10 plot(squeeze(R(1,k,:)),squeeze(R(3,k,:)),['-' mk{mod(k-1,length(mk))+1}],'Color',co(k,:),'MarkerSize',6,'MarkerFaceColor',co(k,:),'DisplayName',sprintf('Nf=%s',pl_str{k})); end end xlabel('Blocking (%)'); ylabel('Dyn Access (%)'); title('Fig6: Dyn Ch Access'); legend('Location','best'); box on; subplot(3,3,6); hold on; grid on; for k=1:n_plots if fixed_caps(k)>0 && fixed_caps(k)<10 plot(squeeze(R(1,k,:)),squeeze(R(4,k,:)),['-' mk{mod(k-1,length(mk))+1}],'Color',co(k,:),'MarkerSize',6,'MarkerFaceColor',co(k,:),'DisplayName',sprintf('Nf=%s',pl_str{k})); end end xlabel('Blocking (%)'); ylabel('Switched (%)'); title('Fig7: Switching Activity'); legend('Location','best'); box on; subplot(3,3,7); hold on; grid on; for k=1:n_plots plot(squeeze(R(1,k,:)),squeeze(R(5,k,:)),['-' mk{mod(k-1,length(mk))+1}],'Color',co(k,:),'MarkerSize',6,'MarkerFaceColor',co(k,:),'DisplayName',sprintf('Nf=%s',pl_str{k})); end xlabel('Blocking (%)'); ylabel('Occupancy (%)'); title('Fig8: Ch Occupancy'); legend('Location','best'); box on; % Subplot 8: Table I subplot(3,3,8); axis off; if ~isempty(tableI_data) t_str = {'TABLE I: Channels Required'; ''; 'Block% FIXSYS Hybrid(8+2)'}; for r = 1:size(tableI_data,1) t_str{end+1} = sprintf(' %2d%% %2d', tableI_data(r,1), tableI_data(r,2)); end t_str{end+1} = ''; t_str{end+1} = 'Hybrid: 8 fixed + 2 dynamic'; t_str{end+1} = 'FIXSYS: 10 fixed + 0 dynamic'; text(0.1, 0.9, t_str, 'FontSize', 10, 'VerticalAlignment', 'top', 'FontName', 'Courier'); end end